实验8 音频信号的时域、频域观测与分析

202300120273

- 实验目的

(1)了解离散时间系统复频域分析的基本实现方法

(2)掌握相关函数的调用格式及作用。

二、实验设备

(1)计算机。

(2)MATLAB软件

三、实验内容

（1）、用 MATLAB 读入一段声音信号,观察其幅频特征和相频特征,并分别设计实现。(1)对声音信号时域压缩，观察其幅频特性的变换;

(2)对声音信号时域开展，观察其幅频特性的变换。

Matlab代码（step函数）

% 读入声音信号 [signal, fs] = audioread('知更鸟,HOYO-MiX,Chevy - 希望有羽毛和翅膀.mp3'); % 如果是立体声，选择一个通道 if size(signal, 2) > 1 signal = signal(:, 1); end % 计算FFT N = length(signal); % 信号长度 Y = fft(signal); % 计算双侧频谱P2 P2 = abs(Y/N); % 单边频谱P1 P1 = P2(1:N/2+1); P1(2:end-1) = 2*P1(2:end-1); % 频率向量 f = fs*(0:(N/2))/N; % 定义压缩或扩展的比例 compression_ratio = 0.5; % 压缩到原来的一半 expansion_ratio = 2; % 扩展到原来的两倍 % 时域压缩 compressed_signal = signal(1:round(1/compression_ratio):end); % 时域扩展 expanded_signal = interp1(1:length(signal), signal, 1:expansion_ratio:length(signal)); N_compressed = length(compressed_signal); Y_compressed = fft(compressed_signal); P2_compressed = abs(Y_compressed/N_compressed); P1_compressed = P2_compressed(1:N_compressed/2+1); P1_compressed(2:end-1) = 2*P1_compressed(2:end-1); f_compressed = fs*(0:(N_compressed/2))/N_compressed; N_expanded = length(expanded_signal); Y_expanded = fft(expanded_signal); P2_expanded = abs(Y_expanded/N_expanded); P1_expanded = P2_expanded(1:N_expanded/2+1); P1_expanded(2:end-1) = 2*P1_expanded(2:end-1); f_expanded = fs*(0:(N_expanded/2))/N_expanded; % 绘制原始、压缩和扩展后的幅频特性图 figure; subplot(3,1,1); plot(f, P1); title('原始信号的单边幅频特性'); xlabel('频率 (Hz)'); ylabel('¦P1(f)¦'); subplot(3,1,2); plot(f_compressed, P1_compressed); title('压缩后信号的单边幅频特性'); xlabel('频率 (Hz)'); ylabel('¦P1(f)¦'); subplot(3,1,3); plot(f_expanded, P1_expanded); title('扩展后信号的单边幅频特性'); xlabel('频率 (Hz)'); ylabel('¦P1(f)¦'); saveas(gcf,"signal_1.jpg");

2、用 MATLAB 读入一幅图像，观察其幅频特征和相频特征，并分别设计实现。

(1)仅由图像频谱的幅度谱恢复图像;

(2)仅由图像频谱的相位谱恢复图像;

Matlab代码（step函数）

% 读入图像 img = imread('1.jpg'); img_gray = rgb2gray(img); % 转换为灰度图像 % 计算2D FFT F = fftshift(fft2(double(img_gray))); % 幅度谱和相位谱 magnitude_spectrum = log(1 + abs(F)); % 对数尺度显示 phase_spectrum = angle(F); fr=real(ifft2(ifftshift(F))); %频率域反变换到空间域，并取实部 ret = im2uint8(mat2gray(fr)); %更改图像类型 % 显示原始图像、幅度谱和相位谱 figure; subplot(1,5,1), imshow(img_gray), title('Original Image'); subplot(1,5,2), imshow(magnitude_spectrum, []), title('Magnitude Spectrum'); subplot(1,5,3), imshow(phase_spectrum, []), title('Phase Spectrum'); subplot(1,5,4), imshow(ret), title('iFourier(Magnitude Spectrum)'); saveas(gcf,"signal_2-1.jpg"); % 构造全1的相位谱或全0的幅度谱 [F_magnitude, F_phase] = cart2pol(real(F), imag(F)); F_phase_only = pol2cart(F_magnitude, ones(size(F_phase))); F_magnitude_only = pol2cart(ones(size(F_magnitude)), F_phase); % 逆FFT以恢复图像 recovered_from_magnitude = ifft2(ifftshift(F_phase_only)); recovered_from_phase = ifft2(ifftshift(F_magnitude_only)); % 显示恢复的图像 figure; subplot(1,2,1), imshow(abs(real(recovered_from_magnitude)), []), title('Recovered from Phase '); subplot(1,2,2), imshow(angle(recovered_from_phase), []), title('Recovered from Magnitude'); saveas(gcf,"signal_2-2.jpg");

幅度谱与相位谱：