信息科学与工程学院

2024－2025学年第二学期

实 验 报 告

课程名称：   FPGA与电子设计自动化

实验名称： 状态机电路的FPGA实现

专  业  班  级    通信工程 三班

学  生  学  号

学  生  姓  名

实  验  时  间   2025年4月 10 日

实验报告

【实验目的】

熟悉Verilog的时序逻辑电路实现

【实验要求】

- 实验内容与原理说明（包括框图、逻辑表达式和真值表）；

- 实验模块程序代码（设计模块Design Block）和激励代码（激励模块Test Bench）；

- 仿真波形图；

- 综合得到的门级电路图；

- 实验结果分析及思考。

【实验具体内容】

实验四  有限状态机设计

- 设计一个交通红绿灯控制器模块，实现主干道和支路之间红绿黄灯的信号转换。

逻辑表达式

1.状态转移条件：

S0 → S1: Tl & S

S1 → S2: Ty

S2 → S3: Ts | ~S

S3 → S0: Ty

2.输出逻辑：

S0: HG=1, FR=1

S1: HY=1, FR=1

S2: HR=1, FG=1

S3: HR=1, FY=1

3.计时器逻辑：

真值表

| 当前状态 | 条件 (Tl, Ts, Ty, S) | 下一状态 | St | HG | HY | HR | FG | FY | FR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0 | Tl=1, S=1 | S1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| S0 | 其他情况 | S0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| S1 | Ty=1 | S2 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| S1 | Ty=0 | S1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| S2 | Ts=1 或 S=0 | S3 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| S2 | 其他情况 | S2 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| S3 | Ty=1 | S0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| S3 | Ty=0 | S3 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |

Design Block 代码

- module controller (CLK, S, nRESET, HG, HY, HR, FG, FY, FR, TimerH, TimerL);

- //gray

- parameter S0=2'b00;

- parameter S1=2'b01;

- parameter S2=2'b11;

- parameter S3=2'b10;

- //one-hot

- // parameter S0=4'b0001;

- // parameter S1=4'b0010;

- // parameter S2=4'b0100;

- // parameter S3=4'b1000;

- //I/O ports

- input CLK, S, nRESET;

- //if S=1, indicates that there is car on the country road

- output HG, HY, HR, FG, FY, FR;

- //declared output signals are registers

- reg    HG, HY, HR, FG, FY, FR;

- output [3:0] TimerH;

- output [3:0] TimerL;

- reg    [3:0] TimerH, TimerL;

- //Internal state variables

- wire Tl, Ts, Ty; //timer output signals

- reg St;               //state translate signal

- reg [1:0] CurrentState, NextState;    //FSM state register

- always @(posedge CLK or negedge nRESET )

- begin:  counter

- if (~nRESET)  {TimerH, TimerL} <= 8'h00;

- else if (St)         {TimerH, TimerL} <= 8'h00;

- else if ((TimerH == 5) & (TimerL == 9))

- begin {TimerH, TimerL} <= {TimerH, TimerL}; end

- else if (TimerL == 9)

- begin TimerH <= TimerH + 1;  TimerL <= 0; end

- else

- begin TimerH <= TimerH; TimerL <= TimerL + 1; end

- end  // BCD counter

- assign  Ty = (TimerH==0)&(TimerL==4);

- assign  Ts = (TimerH==2)&(TimerL==9);

- assign  Tl = (TimerH==5)&(TimerL==9);

- always @(posedge CLK or negedge nRESET )

- begin:  statereg

- if (~nRESET)     //Signal controller starts in S0 state

- CurrentState  <=  S0;

- else      CurrentState  <=  NextState;

- end   //statereg

- // FSM combinational block: state machine using case statements

- always @(S or CurrentState or Tl or Ts or Ty )

- begin: fsm

- case(CurrentState)

- S0: begin

- //S0是用define定义的，在引用时要加右撇号(反撇号)

- NextState = (Tl && S) ? S1 :S0;

- St = (Tl && S) ? 1:0;

- end

- S1:       begin

- NextState = (Ty) ? S2 :S1;

- St = (Ty) ? 1:0;

- end

- S2:       begin

- NextState = (Ts || ~S) ? S3 :S2;

- St = (Ts || ~S) ? 1:0;

- end

- S3:       begin

- NextState = (Ty) ? S0 :S3;

- St = (Ty) ? 1:0;

- end

- endcase

- end  //fsm

- /*===== Description of the decoder block =====*/

- //Compute values of main signal and country signal

- always @(CurrentState)

- begin

- case (CurrentState)

- S0: begin

- {HG, HY, HR} = 3'b100; //Highway signal is green

- {FG, FY, FR}   = 3'b001; //Country signal is red

- end

- S1: begin

- {HG, HY, HR} = 3'b010; //Highway signal is yellow

- {FG, FY, FR}  = 3'b001;    //Country signal is red

- end

- S2: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b100;   //Country signal is green

- end

- S3: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b010;   //Country signal is yellow

- end

- endcase

- end

- endmodule

（One-hot编码）

- module controller (CLK, S, nRESET, HG, HY, HR, FG, FY, FR, TimerH, TimerL);

- //one-hot

- parameter [3:0]S0 = 4'b0001,

- S1 = 4'b0010,

- S2 = 4'b0100,

- S3 = 4'b1000;

- parameter [1:0]S0_POS = 2'd0,

- S1_POS = 2'd1,

- S2_POS = 2'd2,

- S3_POS = 2'd3;

- //I/O ports

- input CLK, S, nRESET;

- //if S=1, indicates that there is car on the country road

- output HG, HY, HR, FG, FY, FR;

- //declared output signals are registers

- reg    HG, HY, HR, FG, FY, FR;

- output [3:0] TimerH;

- output [3:0] TimerL;

- reg    [3:0] TimerH, TimerL;

- //Internal state variables

- wire Tl, Ts, Ty; //timer output signals

- reg St;               //state translate signal

- reg [3:0] CurrentState, NextState;    //FSM state register

- always @(posedge CLK or negedge nRESET )

- begin:  counter

- if (~nRESET)  {TimerH, TimerL} <= 8'h00;

- else if (St)         {TimerH, TimerL} <= 8'h00;

- else if ((TimerH == 5) & (TimerL == 9))

- begin {TimerH, TimerL} <= {TimerH, TimerL}; end

- else if (TimerL == 9)

- begin TimerH <= TimerH + 1;  TimerL <= 0; end

- else

- begin TimerH <= TimerH; TimerL <= TimerL + 1; end

- end  // BCD counter

- assign  Ty = (TimerH==0)&(TimerL==4);

- assign  Ts = (TimerH==2)&(TimerL==9);

- assign  Tl = (TimerH==5)&(TimerL==9);

- always @(posedge CLK or negedge nRESET )

- begin:  statereg

- if (~nRESET)     //Signal controller starts in S0 state

- CurrentState  <=  S0;

- else      CurrentState  <=  NextState;

- end   //statereg

- // FSM combinational block: state machine using case statements

- always @(S or CurrentState or Tl or Ts or Ty )

- begin: fsm

- NextState = S0;

- case(1'b1)

- CurrentState[S0_POS]:

- begin

- NextState = (Tl && S) ? S1 :S0;

- St = (Tl && S) ? 1:0;

- end

- CurrentState[S1_POS]:

- begin

- NextState = (Ty) ? S2 :S1;

- St = (Ty) ? 1:0;

- end

- CurrentState[S2_POS]:

- begin

- NextState = (Ts || ~S) ? S3 :S2;

- St = (Ts || ~S) ? 1:0;

- end

- CurrentState[S3_POS]:

- begin

- NextState = (Ty) ? S0 :S3;

- St = (Ty) ? 1:0;

- end

- endcase

- end  //fsm

- /*===== Description of the decoder block =====*/

- //Compute values of main signal and country signal

- always @(CurrentState)

- begin

- case (CurrentState)

- S0: begin

- {HG, HY, HR} = 3'b100; //Highway signal is green

- {FG, FY, FR}   = 3'b001; //Country signal is red

- end

- S1: begin

- {HG, HY, HR} = 3'b010; //Highway signal is yellow

- {FG, FY, FR}  = 3'b001;    //Country signal is red

- end

- S2: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b100;   //Country signal is green

- end

- S3: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b010;   //Country signal is yellow

- end

- endcase

- end

- endmodule

Test Bench代码

- `timescale 1ns/1ns

- module controller_tb;

- // 输入信号

- reg CLK;

- reg S;

- reg nRESET;

- // 输出信号

- wire HG, HY, HR, FG, FY, FR;

- wire [3:0] TimerH;

- wire [3:0] TimerL;

- // 实例化被测模块（UUT）

- controller uut (

- .CLK(CLK),

- .S(S),

- .nRESET(nRESET),

- .HG(HG),

- .HY(HY),

- .HR(HR),

- .FG(FG),

- .FY(FY),

- .FR(FR),

- .TimerH(TimerH),

- .TimerL(TimerL)

- );

- // 生成时钟信号（周期10ns）

- initial begin

- CLK = 0;

- forever #5 CLK = ~CLK; // 每5ns翻转，形成10ns周期

- end

- // 生成复位信号

- initial begin

- nRESET = 0;   // 初始复位有效

- #20;          // 保持20ns

- nRESET = 1;   // 撤销复位

- end

- // 注意：仿真中1个时间单位代表100ms，此处简化为1ns代替实际100ms

- // 测试激励与用例

- initial begin

- // 初始化输入

- S = 0;

- // 测试用例1：验证复位状态

- #20; // 等待复位完成

- // 测试用例2：S=0时保持S0状态

- #590; // 等待59个周期（590ns）

- // 为下一测试重置系统

- nRESET = 0;

- #10;

- nRESET = 1;

- #10;

- // 测试用例3：S=1触发S0->S1转换

- S = 1;

- #590; // 等待59秒（仿真中590ns）

- #10;  // 确保状态转换

- // 测试用例4：验证S1->S2转换（4秒超时）

- #40;  // 等待4秒（40ns）

- #10;  // 确保状态转换

- // 测试用例5：S2状态时S=0触发复位

- S = 0;

- @(posedge CLK); // 等待时钟上升沿

- // 重置系统

- nRESET = 0;

- #10;

- nRESET = 1;

- S = 1;

- #590; // 再次进入S1状态

- #10;

- #40;  // 进入S2状态

- #10;

- // 测试用例6：验证S2超时（29秒）转S3

- #290; // 等待29秒（290ns）

- @(posedge CLK); // 同步检查

- // 测试用例7：验证S3->S0转换（4秒黄灯）

- #40;  // 等待4秒（40ns）

- @(posedge CLK); // 同步检查

- end

- endmodule

综合得到门级电路图：

图 1 gray码

图 2 one-hot 编码

这里我们选择的器件为：EP3C40Q240C8，它有着39,600个逻辑器件，我们综合得到所耗费的逻辑器件总数为：

图 3 gray码

图 4 one-hot编码

可以发现one-hot编码相对于gray的复杂逻辑资源消耗得少，但这里两者总寄存器资源消耗相当，one-hot略少于gray码。

仿真波形图：

图 5 60秒绿灯

图 6 4秒黄灯

以上逻辑均满足，以达到要求。

- 设计一个小轿车尾灯控制器模块（以书中的例子）。

逻辑表达式：

- 状态转移逻辑：

IDLE → WARN: HAZ | (LEFT & RIGHT)

IDLE → LEF: ~HAZ & LEFT & ~(LEFT & RIGHT)

IDLE → RIG: ~HAZ & RIGHT & ~(LEFT & RIGHT)

LEF → WARN: HAZ

RIG → WARN: HAZ

WARN → IDLE: 无条件

- 输出逻辑：

IDLE: L=3'b000, R=3'b000

LEF: L = ~L_prev（翻转），R=3'b000

RIG: R = ~R_prev（翻转），L=3'b000

WARN: L=3'b111, R=3'b111

真值表：

IDLE 状态输入组合

| HAZ | LEFT | RIGHT | Next State |
| --- | --- | --- | --- |
| 0 | 0 | 0 | IDLE |
| 0 | 0 | 1 | RIG |
| 0 | 1 | 0 | LEF |
| 0 | 1 | 1 | WARN |
| 1 | 0 | 0 | WARN |
| 1 | 0 | 1 | WARN |
| 1 | 1 | 0 | WARN |
| 1 | 1 | 1 | LEF |

LEF 状态输入组合

| HAZ | LEFT | Next State |
| --- | --- | --- |
| 0 | 0 | IDLE |
| 0 | 1 | LEF |
| 1 | 0 | WARN |
| 1 | 1 | WARN |

RIG 状态输入组合

| HAZ | RIGHT | Next State |
| --- | --- | --- |
| 0 | 0 | IDLE |
| 0 | 1 | RIG |
| 1 | 0 | WARN |
| 1 | 1 | WARN |

输出真值表

| Current State | L | R |
| --- | --- | --- |
| IDLE | 000 | 000 |
| LEF | ~L_prev | 000 |
| RIG | 000 | ~R_prev |
| WARN | 111 | 111 |

Design Block 代码

- module car_led_con(CLK,HAZ,LEFT,RIGHT,nCR,L,R);

- //格雷码编码

- parameter IDLE = 2'b00;

- parameter LEF = 2'b01;

- parameter RIG = 2'b11;

- parameter WARN = 2'b10;

- //one-hot编码

- // parameter IDLE = 4'b0001;

- // parameter LEF = 4'b0010;

- // parameter RIG = 4'b0100;

- // parameter WARN = 4'b1000;

- input CLK,HAZ,LEFT,RIGHT,nCR;

- output reg [2:0]L;

- output reg [2:0]R;

- reg [1:0]Current_State;

- reg [1:0]Next_State;

- always@(posedge CLK or negedge nCR)

- begin

- if(~nCR)

- Current_State <= IDLE;

- else

- Current_State <= Next_State;

- end

- always@(Current_State or HAZ or LEFT or RIGHT)

- begin

- Next_State = 2'b00;

- case(Current_State)

- IDLE:

- begin

- L = 3'b000;

- R = 3'b000;

- Next_State = IDLE;

- if(HAZ + LEFT*RIGHT ==1)

- Next_State = WARN;

- if(LEFT == 1)

- Next_State = LEF;

- if(RIGHT == 1)

- Next_State = RIG;

- end

- LEF://左转向灯和右转向灯不可能直接切换

- begin

- L = ~L;

- Next_State = IDLE;

- if(LEFT == 1)

- Next_State = LEF;

- if(HAZ == 1)

- Next_State = WARN;

- end

- RIG:

- begin

- R = ~R;

- Next_State = IDLE;

- if(RIGHT == 1)

- Next_State = RIG;

- if(HAZ == 1)

- Next_State = WARN;

- end

- WARN:

- begin

- L = 3'd7;

- R = 3'd7;

- Next_State = IDLE;

- end

- endcase

- end

- endmodule

(one-hot编码)

- module car_led_con(CLK,HAZ,LEFT,RIGHT,nCR,L,R);

- input CLK,HAZ,LEFT,RIGHT,nCR;

- output reg [2:0]L;

- output reg [2:0]R;

- reg [3:0]state,next_state;

- parameter [3:0] IDLE = 4'b0001,

- LEF = 4'b0010,

- RIG = 4'b0100,

- WARN = 4'b1000;

- parameter [1:0] IDLE_POS = 2'd0,

- LEF_POS = 2'd1,

- RIG_POS = 2'd2,

- WARN_POS = 2'd3;

- always@(posedge CLK or negedge nCR)

- begin

- if(~nCR)

- state <= IDLE;

- else

- state <= next_state;

- end

- always@(HAZ or LEFT or RIGHT)

- begin

- next_state = IDLE;

- case(1'b1)

- state[IDLE_POS]:

- begin

- if(HAZ + LEFT*RIGHT ==1)

- next_state = WARN;

- if(LEFT == 1)

- next_state = LEF;

- if(RIGHT == 1)

- next_state = RIG;

- if(~(HAZ+RIGHT+LEFT))

- next_state = IDLE;

- end

- state[LEF_POS]:

- begin

- next_state = IDLE;

- if(HAZ == 1)

- next_state = WARN;

- end

- state[RIG_POS]:

- begin

- next_state = IDLE;

- if(HAZ == 1)

- next_state = WARN;

- end

- state[IDLE_POS]:

- begin

- next_state = IDLE;

- end

- endcase

- end

- always@(state)

- begin

- case(state)

- IDLE:begin

- L = 3'b0;

- R = 3'b0;

- end

- LEF:begin

- L = 3'd7;

- R = 3'b0;

- end

- RIG:begin

- L = 3'b0;

- R = 3'd7;

- end

- WARN:begin

- L = 3'd7;

- R = 3'd7;

- end

- endcase

- end

- endmodule

Test Bench代码

- `timescale 1ns / 1ps

- module car_led_con_tb;

- // 输入信号

- reg CLK;

- reg HAZ;

- reg LEFT;

- reg RIGHT;

- reg nCR;

- // 输出信号

- wire [2:0] L;

- wire [2:0] R;

- // 实例化被测模块

- car_led_con uut (

- .CLK(CLK),

- .HAZ(HAZ),

- .LEFT(LEFT),

- .RIGHT(RIGHT),

- .nCR(nCR),

- .L(L),

- .R(R)

- );

- // 生成时钟信号，周期10ns

- initial begin

- CLK = 0;

- forever #5 CLK = ~CLK;

- end

- initial begin

- // 初始化输入

- HAZ = 0;

- LEFT = 0;

- RIGHT = 0;

- nCR = 0;  // 初始复位

- // 测试复位功能

- #10; // 等待一个时钟周期

- if (L !== 3'b000 || R !== 3'b000)

- $display("Error: Reset failed. L=%b, R=%b", L, R);

- else

- $display("Reset test passed.");

- nCR = 1; // 释放复位

- #10;

- // 测试危险信号（HAZ）

- HAZ = 1;

- #10; // 进入WARN状态

- if (L !== 3'b111 || R !== 3'b111)

- $display("Error: HAZ failed. L=%b, R=%b", L, R);

- else

- $display("HAZ test passed.");

- HAZ = 0;

- // 测试左转（LEFT）

- LEFT = 1;

- #10; // 进入LEF状态，L翻转

- #10; // 第二个周期，继续LEF，L再次翻转

- LEFT = 0; // 释放LEFT

- #10; // 返回IDLE

- // 测试右转（RIGHT）

- RIGHT = 1;

- #10; // 进入RIG状态，R翻转

- #10; // 第二个周期，继续RIG，R再次翻转

- RIGHT = 0; // 释放RIGHT

- #10; // 返回IDLE

- // 测试同时按下LEFT和RIGHT

- LEFT = 1;

- RIGHT = 1;

- #10; // 应进入WARN状态

- LEFT = 0;

- RIGHT = 0;

- #10; // 返回IDLE

- // 测试在左转时触发HAZ

- LEFT = 1;

- #10; // 进入LEF

- HAZ = 1; // 触发危险信号

- #10; // 应进入WARN

- HAZ = 0;

- #10; // 返回IDLE

- end

- endmodule

综合得到门级电路图：

图 7 gray

图 8 one-hot

综合所耗费资源：

图 9 gray

图 10 one-hot

可以发现one-hot编码相对于gray的复杂逻辑资源消耗得少，但这里两者总寄存器资源消耗相当，总资源one-hot少于gray码。

仿真波形图：

图 11 刹车踩下

两个灯双闪，成功

图 12 左转启动

单左转灯闪动，成功

图 13 右转与刹车同时

两个灯双闪，成功

综上，功能完成。

- 设计一个10层楼的电梯控制器模块，要求：以楼层最短位置先后优先级进行设计。

逻辑表达式：

- 符号定义：

- 清除请求逻辑：

- 目标楼层选择

- 移动方向抉择：

Design Block 代码(one-hot)

- module elevator_controller (

- input        clk,

- input        rst_n,

- input  [9:0] req_floors, //one-hot 编码

- output reg [3:0] current_floor,

- output reg [9:0] current_req,

- output reg      moving_up,

- output reg      moving_down

- );

- // 请求寄存器与清除请求标志

- // reg [9:0] current_req;

- reg [9:0] next_req;

- reg [3:0] next_floor;

- reg       clear_request;      // 请求清除标志

- reg [3:0] clear_floor;        // 待清除的楼层

- always @(posedge clk or negedge rst_n)

- begin

- if (!rst_n)

- begin

- current_req   <= 10'b0;     // 复位清零

- current_floor <= 4'd1;

- end

- else

- begin

- current_req       <= next_req;

- current_floor     <= next_floor;

- end

- end

- always@(current_floor or current_req or req_floors) begin: movement_logic

- integer i, min_dist, nearest_floor,dist;

- next_req = current_req | req_floors;

- if (current_req[current_floor])

- begin

- next_req[current_floor] = 0;

- moving_up     = 1'b0;

- moving_down   = 1'b0;

- end

- else

- begin

- min_dist = 10;

- nearest_floor = current_floor; // 默认为原地踏步

- for (i = 0; i < 10; i = i+1)

- begin

- if (current_req[i])

- begin

- dist = (i > current_floor) ?

- (i - current_floor) :

- (current_floor - i);

- if (dist < min_dist)

- begin

- min_dist      = dist;

- nearest_floor = i;

- end

- else if (dist == min_dist && i > nearest_floor)

- nearest_floor = i;    //优先向上

- end

- end

- if (nearest_floor != current_floor) begin

- if (nearest_floor > current_floor) begin

- next_floor = current_floor + 1;

- moving_up     = 1'b1;

- moving_down   = 1'b0;

- end else begin

- next_floor = current_floor - 1;

- moving_up     = 1'b0;

- moving_down   = 1'b1;

- end

- end

- else //若无指令，则原地等待

- begin

- next_floor = current_floor;

- moving_up   = 1'b0;

- moving_down = 1'b0;

- end

- end

- end

- endmodule

Test Bench代码

- `timescale 1ns/1ps

- module tb_elevator_controller();

- reg clk;

- reg rst_n;

- reg [9:0] req_floors;

- wire [3:0] current_floor;

- wire moving_up;

- wire moving_down;

- elevator_controller dut (

- .clk(clk),

- .rst_n(rst_n),

- .req_floors(req_floors),

- .current_floor(current_floor),

- .moving_up(moving_up),

- .moving_down(moving_down)

- );

- // 生成时钟信号

- initial begin

- clk = 0;

- forever #5 clk = ~clk;

- end

- // 生成复位信号

- initial begin

- rst_n = 0;

- #20 rst_n = 1;

- end

- // 测试用例

- initial begin

- req_floors = 10'b0000000000;

- #25; // 等待复位完成

- // 测试用例1：检查复位状态

- if (current_floor !== 4'd1) $display("Test 1 Failed: Current floor not 1 after reset!");

- else if (moving_up || moving_down) $display("Test 1 Failed: Moving after reset!");

- else $display("Test 1 Passed: Reset OK.");

- // 测试用例2：单层请求（1 -> 5）

- #5;

- req_floors = 10'b0000100000; // 请求楼层5

- wait (current_floor == 4'd5); // 等待到达5层

- #1 req_floors = 10'b0; // 清除请求

- #10; // 等待状态更新

- if (current_floor !== 4'd5) $display("Test 2 Failed: Didn't reach floor 5!");

- else if (moving_up || moving_down) $display("Test 2 Failed: Didn't stop at floor 5!");

- else $display("Test 2 Passed: Reached floor 5 and stopped.");

- // 测试用例3：多层请求（5 -> 3 -> 1）

- #20;

- req_floors = 10'b0000001000; // 请求楼层3

- wait (current_floor == 4'd3);

- #1 req_floors = 10'b0;

- #10;

- req_floors = 10'b0000100010; // 同时请求1层和5层

- wait (current_floor == 4'd5); // 应优先到5层

- #1 req_floors = 10'b0000000010; // 清除5层请求

- wait (current_floor == 4'd1); // 然后到1层

- #1 req_floors = 10'b0;

- #10;

- if (current_floor !== 4'd1) $display("Test 3 Failed: Didn't reach floor 1!");

- else if (moving_up || moving_down) $display("Test 3 Failed: Didn't stop at floor 1!");

- else $display("Test 3 Passed: Handled multiple requests.");

- // 测试用例4：同距离请求优先向上（3层请求2和4）

- #20;

- req_floors = 10'b0000001100; // 请求楼层2和3

- wait (current_floor == 4'd2);

- #1 req_floors = 10'b0000001000;

- wait (current_floor == 4'd3);

- #1 req_floors = 10'b0000011000; // 当前在3层，请求2和4层

- wait (current_floor == 4'd4); // 应优先到4层

- #1 req_floors = 10'b0000001000;

- wait (current_floor == 4'd2);

- #1 req_floors = 10'b0;

- #10;

- if (current_floor !== 4'd2) $display("Test 4 Failed: Priority error!");

- else $display("Test 4 Passed: Upward priority handled.");

- $finish;

- end

- // 监视关键信号

- initial begin

- $monitor("Time: %0tns | Floor: %0d | Up: %b | Down: %b | Requests: %b",

- $time, current_floor, moving_up, moving_down, req_floors);

- end

- endmodule

Design Block（Gray码编写）

- module elevator_controller (

- input        clk,         // 时钟信号

- input        rst_n,       // 异步复位信号（低有效）

- input  [3:0] req_floors,  // 4位格雷码输入（楼层请求）

- output reg [3:0] current_floor, // 当前楼层（二进制编码）

- output reg [9:0] current_req,   // 当前所有楼层请求（one-hot编码）

- output reg      moving_up,      // 上行指示灯

- output reg      moving_down     // 下行指示灯

- );

- function [9:0] gray_to_onehot;

- input [3:0] gray;

- reg [3:0] binary;  // 中间二进制值

- begin

- // Gray码转二进制（逐位异或）

- binary[3] = gray[3];

- binary[2] = gray[2] ^ binary[3];

- binary[1] = gray[1] ^ binary[2];

- binary[0] = gray[0] ^ binary[1];

- // 二进制转one-hot（仅处理0-9楼层）

- if (binary <= 4'd9)

- gray_to_onehot = 10'b1 << binary;  // 左移生成one-hot

- else

- gray_to_onehot = 10'b0;           // 无效值处理

- end

- endfunction

- reg [9:0] next_req;    // 下一时刻请求寄存器

- reg [3:0] next_floor;  // 下一时刻楼层寄存器

- // 时序逻辑：寄存器更新

- always @(posedge clk or negedge rst_n) begin

- if (!rst_n) begin              // 复位初始化

- current_req   <= 10'b0;     // 清空所有请求

- current_floor <= 4'd1;     // 初始楼层设为1层

- end else begin

- current_req   <= next_req;   // 更新请求寄存器

- current_floor <= next_floor; // 更新楼层寄存器

- end

- end

- always @(*) begin

- integer i;                      // 循环计数器

- integer min_dist;               // 最小距离

- integer nearest_floor;          // 最近目标楼层

- integer dist;                   // 临时距离存储

- reg [9:0] req_combined;         // 合并后的请求信号

- // 请求合并：当前请求 + 新Gray码转换请求

- req_combined = current_req | gray_to_onehot(req_floors);

- next_req = req_combined;        // 默认保持请求

- // 当前楼层有请求时处理

- if (req_combined[current_floor]) begin

- next_req[current_floor] = 1'b0;  // 清除当前楼层请求

- next_floor = current_floor;      // 保持当前楼层

- moving_up   = 1'b0;              // 停止移动

- moving_down = 1'b0;

- end

- // 无当前楼层请求时寻找下一个目标

- else begin

- min_dist = 10;                   // 初始最小距离（最大为9）

- nearest_floor = current_floor;    // 默认保持不动

- // 遍历所有楼层寻找最近请求

- for (i = 0; i < 10; i = i+1) begin

- if (req_combined[i]) begin    // 检测到楼层i有请求

- // 计算绝对距离

- dist = (i > current_floor) ?

- (i - current_floor) :

- (current_floor - i);

- // 更新最近楼层逻辑

- if (dist < min_dist) begin       // 发现更近楼层

- min_dist = dist;

- nearest_floor = i;

- end

- // 距离相同时优先上行

- else if ((dist == min_dist) && (i > nearest_floor)) begin

- nearest_floor = i;

- end

- end

- end

- // 移动方向决策

- if (nearest_floor != current_floor) begin

- // 上行决策

- if (nearest_floor > current_floor) begin

- next_floor = current_floor + 1;  // 楼层+1

- moving_up   = 1'b1;             // 点亮上行灯

- moving_down = 1'b0;

- end

- // 下行决策

- else begin

- next_floor = current_floor - 1;  // 楼层-1

- moving_up   = 1'b0;

- moving_down = 1'b1;              // 点亮下行灯

- end

- end

- // 无请求时保持静止

- else begin

- next_floor = current_floor;

- moving_up   = 1'b0;

- moving_down = 1'b0;

- end

- end

- end

- endmodule

综合得到门级电路图：

图 14 one-hot

图 15 gray

综合所耗费资源如下：

图 16 one-hot

图 17 gray

可以发现one-hot编码相对于gray的复杂逻辑资源消耗得少，但这里两者总寄存器资源消耗相当，one-hot略少于gray码。

仿真波形图如下：

图 18 向上6楼逻辑验证

成功上到6楼

图 19 优先距离逻辑测试

成功优先上到6楼，再下2楼

图 20 相同距离优先向上逻辑测试

成功优先上到5楼，再下3楼

综上，控制逻辑验证成功。

【实验心得与思考】：

在FPGA设计中，格雷编码状态机与One-Hot编码状态机的资源消耗差异主要体现在触发器和组合逻辑的权衡上。格雷码通过log2(N)个触发器实现N个状态，触发器资源占用少且随状态数增长缓慢，适合大规模状态机；但其状态转换需额外的组合逻辑（如格雷码与二进制转换），可能导致LUT资源消耗增加。而One-Hot编码每个状态独占一个触发器（共需N个），触发器开销较大，但组合逻辑通常更简单（仅需激活单个状态位），LUT利用率更低，尤其在状态转移条件简单时更具优势。此外，One-Hot因独立布线可能占用更多路径资源，但现代FPGA的布线能力对此影响较小。综合来看，若设计侧重节省触发器（如状态数多或低端器件），格雷码更优；若追求组合逻辑简化或状态数较少（如≤8），One-Hot更高效，且其结构易被综合工具优化。实际选择需结合目标FPGA的资源分布（触发器/LUT比例）和时序需求灵活权衡。