信息科学与工程学院

2024－2025学年第二学期

实 验 报 告

课程名称：   FPGA与电子设计自动化

实验名称： 状态机电路的FPGA实现

专  业  班  级    通信工程 三班

学  生  学  号

学  生  姓  名

实  验  时  间   2025年4月 10 日

实验报告

【实验目的】

熟悉Verilog的时序逻辑电路实现

【实验要求】

- 实验内容与原理说明（包括框图、逻辑表达式和真值表）；

- 实验模块程序代码（设计模块Design Block）和激励代码（激励模块Test Bench）；

- 仿真波形图；

- 综合得到的门级电路图；

- 实验结果分析及思考。

【实验具体内容】

实验四  有限状态机设计

- 设计一个交通红绿灯控制器模块，实现主干道和支路之间红绿黄灯的信号转换。

逻辑表达式：

Design Block 代码

- module controller (CLK, S, nRESET, HG, HY, HR, FG, FY, FR, TimerH, TimerL);

- //gray

- parameter S0=2'b00;

- parameter S1=2'b01;

- parameter S2=2'b11;

- parameter S3=2'b10;

- //one-hot

- // parameter S0=4'b0001;

- // parameter S1=4'b0010;

- // parameter S2=4'b0100;

- // parameter S3=4'b1000;

- //I/O ports

- input CLK, S, nRESET;

- //if S=1, indicates that there is car on the country road

- output HG, HY, HR, FG, FY, FR;

- //declared output signals are registers

- reg    HG, HY, HR, FG, FY, FR;

- output [3:0] TimerH;

- output [3:0] TimerL;

- reg    [3:0] TimerH, TimerL;

- //Internal state variables

- wire Tl, Ts, Ty; //timer output signals

- reg St;               //state translate signal

- reg [1:0] CurrentState, NextState;    //FSM state register

- always @(posedge CLK or negedge nRESET )

- begin:  counter

- if (~nRESET)  {TimerH, TimerL} <= 8'h00;

- else if (St)         {TimerH, TimerL} <= 8'h00;

- else if ((TimerH == 5) & (TimerL == 9))

- begin {TimerH, TimerL} <= {TimerH, TimerL}; end

- else if (TimerL == 9)

- begin TimerH <= TimerH + 1;  TimerL <= 0; end

- else

- begin TimerH <= TimerH; TimerL <= TimerL + 1; end

- end  // BCD counter

- assign  Ty = (TimerH==0)&(TimerL==4);

- assign  Ts = (TimerH==2)&(TimerL==9);

- assign  Tl = (TimerH==5)&(TimerL==9);

- always @(posedge CLK or negedge nRESET )

- begin:  statereg

- if (~nRESET)     //Signal controller starts in S0 state

- CurrentState  <=  S0;

- else      CurrentState  <=  NextState;

- end   //statereg

- // FSM combinational block: state machine using case statements

- always @(S or CurrentState or Tl or Ts or Ty )

- begin: fsm

- case(CurrentState)

- S0: begin

- //S0是用define定义的，在引用时要加右撇号(反撇号)

- NextState = (Tl && S) ? S1 :S0;

- St = (Tl && S) ? 1:0;

- end

- S1:       begin

- NextState = (Ty) ? S2 :S1;

- St = (Ty) ? 1:0;

- end

- S2:       begin

- NextState = (Ts || ~S) ? S3 :S2;

- St = (Ts || ~S) ? 1:0;

- end

- S3:       begin

- NextState = (Ty) ? S0 :S3;

- St = (Ty) ? 1:0;

- end

- endcase

- end  //fsm

- /*===== Description of the decoder block =====*/

- //Compute values of main signal and country signal

- always @(CurrentState)

- begin

- case (CurrentState)

- S0: begin

- {HG, HY, HR} = 3'b100; //Highway signal is green

- {FG, FY, FR}   = 3'b001; //Country signal is red

- end

- S1: begin

- {HG, HY, HR} = 3'b010; //Highway signal is yellow

- {FG, FY, FR}  = 3'b001;    //Country signal is red

- end

- S2: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b100;   //Country signal is green

- end

- S3: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b010;   //Country signal is yellow

- end

- endcase

- end

- endmodule

（One-hot编码）

- module controller (CLK, S, nRESET, HG, HY, HR, FG, FY, FR, TimerH, TimerL);

- //one-hot

- parameter [3:0]S0 = 4'b0001,

- S1 = 4'b0010,

- S2 = 4'b0100,

- S3 = 4'b1000;

- parameter [1:0]S0_POS = 2'd0,

- S1_POS = 2'd1,

- S2_POS = 2'd2,

- S3_POS = 2'd3;

- //I/O ports

- input CLK, S, nRESET;

- //if S=1, indicates that there is car on the country road

- output HG, HY, HR, FG, FY, FR;

- //declared output signals are registers

- reg    HG, HY, HR, FG, FY, FR;

- output [3:0] TimerH;

- output [3:0] TimerL;

- reg    [3:0] TimerH, TimerL;

- //Internal state variables

- wire Tl, Ts, Ty; //timer output signals

- reg St;               //state translate signal

- reg [3:0] CurrentState, NextState;    //FSM state register

- always @(posedge CLK or negedge nRESET )

- begin:  counter

- if (~nRESET)  {TimerH, TimerL} <= 8'h00;

- else if (St)         {TimerH, TimerL} <= 8'h00;

- else if ((TimerH == 5) & (TimerL == 9))

- begin {TimerH, TimerL} <= {TimerH, TimerL}; end

- else if (TimerL == 9)

- begin TimerH <= TimerH + 1;  TimerL <= 0; end

- else

- begin TimerH <= TimerH; TimerL <= TimerL + 1; end

- end  // BCD counter

- assign  Ty = (TimerH==0)&(TimerL==4);

- assign  Ts = (TimerH==2)&(TimerL==9);

- assign  Tl = (TimerH==5)&(TimerL==9);

- always @(posedge CLK or negedge nRESET )

- begin:  statereg

- if (~nRESET)     //Signal controller starts in S0 state

- CurrentState  <=  S0;

- else      CurrentState  <=  NextState;

- end   //statereg

- // FSM combinational block: state machine using case statements

- always @(S or CurrentState or Tl or Ts or Ty )

- begin: fsm

- NextState = S0;

- case(1'b1)

- CurrentState[S0_POS]:

- begin

- NextState = (Tl && S) ? S1 :S0;

- St = (Tl && S) ? 1:0;

- end

- CurrentState[S1_POS]:

- begin

- NextState = (Ty) ? S2 :S1;

- St = (Ty) ? 1:0;

- end

- CurrentState[S2_POS]:

- begin

- NextState = (Ts || ~S) ? S3 :S2;

- St = (Ts || ~S) ? 1:0;

- end

- CurrentState[S3_POS]:

- begin

- NextState = (Ty) ? S0 :S3;

- St = (Ty) ? 1:0;

- end

- endcase

- end  //fsm

- /*===== Description of the decoder block =====*/

- //Compute values of main signal and country signal

- always @(CurrentState)

- begin

- case (CurrentState)

- S0: begin

- {HG, HY, HR} = 3'b100; //Highway signal is green

- {FG, FY, FR}   = 3'b001; //Country signal is red

- end

- S1: begin

- {HG, HY, HR} = 3'b010; //Highway signal is yellow

- {FG, FY, FR}  = 3'b001;    //Country signal is red

- end

- S2: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b100;   //Country signal is green

- end

- S3: begin

- {HG, HY, HR} = 3'b001;   //Highway signal is red

- {FG, FY, FR}   = 3'b010;   //Country signal is yellow

- end

- endcase

- end

- endmodule

- 设计一个小轿车尾灯控制器模块（以书中的例子）。

Design Block 代码

- module car_led_con(CLK,HAZ,LEFT,RIGHT,nCR,L,R);

- //格雷码编码

- parameter IDLE = 2'b00;

- parameter LEF = 2'b01;

- parameter RIG = 2'b11;

- parameter WARN = 2'b10;

- //one-hot编码

- // parameter IDLE = 4'b0001;

- // parameter LEF = 4'b0010;

- // parameter RIG = 4'b0100;

- // parameter WARN = 4'b1000;

- input CLK,HAZ,LEFT,RIGHT,nCR;

- output reg [2:0]L;

- output reg [2:0]R;

- reg [1:0]Current_State;

- reg [1:0]Next_State;

- always@(posedge CLK or negedge nCR)

- begin

- if(~nCR)

- Current_State <= IDLE;

- else

- Current_State <= Next_State;

- end

- always@(Current_State or HAZ or LEFT or RIGHT)

- begin

- Next_State = 2'b00;

- case(Current_State)

- IDLE:

- begin

- L = 3'b000;

- R = 3'b000;

- Next_State = IDLE;

- if(HAZ + LEFT*RIGHT ==1)

- Next_State = WARN;

- if(LEFT == 1)

- Next_State = LEF;

- if(RIGHT == 1)

- Next_State = RIG;

- end

- LEF://左转向灯和右转向灯不可能直接切换

- begin

- L = ~L;

- Next_State = IDLE;

- if(LEFT == 1)

- Next_State = LEF;

- if(HAZ == 1)

- Next_State = WARN;

- end

- RIG:

- begin

- R = ~R;

- Next_State = IDLE;

- if(RIGHT == 1)

- Next_State = RIG;

- if(HAZ == 1)

- Next_State = WARN;

- end

- WARN:

- begin

- L = 3'd7;

- R = 3'd7;

- Next_State = IDLE;

- end

- endcase

- end

- endmodule

(one-hot编码)

- module car_led_con(CLK,HAZ,LEFT,RIGHT,nCR,L,R);

- input CLK,HAZ,LEFT,RIGHT,nCR;

- output reg [2:0]L;

- output reg [2:0]R;

- reg [3:0]state,next_state;

- parameter [3:0] IDLE = 4'b0001,

- LEF = 4'b0010,

- RIG = 4'b0100,

- WARN = 4'b1000;

- parameter [1:0] IDLE_POS = 2'd0,

- LEF_POS = 2'd1,

- RIG_POS = 2'd2,

- WARN_POS = 2'd3;

- always@(posedge CLK or negedge nCR)

- begin

- if(~nCR)

- state <= IDLE;

- else

- state <= next_state;

- end

- always@(HAZ or LEFT or RIGHT)

- begin

- next_state = IDLE;

- case(1'b1)

- state[IDLE_POS]:

- begin

- if(HAZ + LEFT*RIGHT ==1)

- next_state = WARN;

- if(LEFT == 1)

- next_state = LEF;

- if(RIGHT == 1)

- next_state = RIG;

- if(~(HAZ+RIGHT+LEFT))

- next_state = IDLE;

- end

- state[LEF_POS]:

- begin

- next_state = IDLE;

- if(HAZ == 1)

- next_state = WARN;

- end

- state[RIG_POS]:

- begin

- next_state = IDLE;

- if(HAZ == 1)

- next_state = WARN;

- end

- state[IDLE_POS]:

- begin

- next_state = IDLE;

- end

- endcase

- end

- always@(state)

- begin

- case(state)

- IDLE:begin

- L = 3'b0;

- R = 3'b0;

- end

- LEF:begin

- L = 3'd7;

- R = 3'b0;

- end

- RIG:begin

- L = 3'b0;

- R = 3'd7;

- end

- WARN:begin

- L = 3'd7;

- R = 3'd7;

- end

- endcase

- end

- endmodule

- 设计一个10层楼的电梯控制器模块，要求：以楼层最短位置先后优先级进行设计。

Design Block 代码(one-hot)

- module elevator_controller (

- input        clk,

- input        rst_n,

- input  [9:0] req_floors, //one-hot 编码

- output reg [3:0] current_floor,

- output reg [9:0] current_req,

- output reg      moving_up,

- output reg      moving_down

- );

- // 请求寄存器与清除请求标志

- // reg [9:0] current_req;

- reg [9:0] next_req;

- reg [3:0] next_floor;

- reg       clear_request;      // 请求清除标志

- reg [3:0] clear_floor;        // 待清除的楼层

- always @(posedge clk or negedge rst_n)

- begin

- if (!rst_n)

- begin

- current_req   <= 10'b0;     // 复位清零

- current_floor <= 4'd1;

- end

- else

- begin

- current_req       <= next_req;

- current_floor     <= next_floor;

- end

- end

- always@(current_floor or current_req or req_floors) begin: movement_logic

- integer i, min_dist, nearest_floor,dist;

- next_req = current_req | req_floors;

- if (current_req[current_floor])

- begin

- next_req[current_floor] = 0;

- moving_up     = 1'b0;

- moving_down   = 1'b0;

- end

- else

- begin

- min_dist = 10;

- nearest_floor = current_floor; // 默认为原地踏步

- for (i = 0; i < 10; i = i+1)

- begin

- if (current_req[i])

- begin

- dist = (i > current_floor) ?

- (i - current_floor) :

- (current_floor - i);

- if (dist < min_dist)

- begin

- min_dist      = dist;

- nearest_floor = i;

- end

- else if (dist == min_dist && i > nearest_floor)

- nearest_floor = i;    //优先向上

- end

- end

- if (nearest_floor != current_floor) begin

- if (nearest_floor > current_floor) begin

- next_floor = current_floor + 1;

- moving_up     = 1'b1;

- moving_down   = 1'b0;

- end else begin

- next_floor = current_floor - 1;

- moving_up     = 1'b0;

- moving_down   = 1'b1;

- end

- end

- else //若无指令，则原地等待

- begin

- next_floor = current_floor;

- moving_up   = 1'b0;

- moving_down = 1'b0;

- end

- end

- end

- endmodule